SUZUKI 650 BURGMAN MOUNTING INSTRUCTIONS

UPPER FRONT:						LOWER REAR:

(4)   8mm - 1.25 x 35 bolt					(1)   8mm -1.25 x 35 bolt

(1)   8mm -1.25 x 25 bolt					(1)   8mm lock washer

(5)   8mm lock washer					(1)   8mm flat washer

LOWER FRONT:						LOWER REAR:

(4)   8mm - 1.25 x 35 bolt					(*) Stock Bolt

(4)   8mm lock washer

SUZUKI 650 BURGMAN MOUNTING INSTRUCTIONS

- Remove engine access shroud.				 6)   Remove lower rear fascia (above mud guard).

- Remove lower handlebar fascia.				 7)   Remove upper rear fascia.

- Remove upper dash.					 8)   Remove right passenger grab rail.

- Remove lower dash.					 9)   Remove right rear quarter panel.

- Remove right lower cowl (panel under right foot rest). 	 10) Remove center stand.

SUZUKI 650 BURGMAN MOUNTING INSTRUCTIONS

SUZUKI 650 BURGMAN MOUNTING INSTRUCTIONS

SUZUKI 650 BURGMAN MOUNTING INSTRUCTIONS

- Install upper front mount using (4) 8mm x 35 bolts with lock washers for the clamp and (1) 8mm x 25 bolt for the brake.

- Install lower front mount using bolts (4) 8mm x 35 bolts with lock washers.  Position the mount approx. 1/8” from left lower cowl.

- Install upper rear mount using stock bolts.  Be sure to position the mount with the triangular gusset to the left.

SUZUKI 650 BURGMAN MOUNTING INSTRUCTIONS

- Remove the center stand

- Install lower rear mount on the left side using the stock center stand bolt and (1) 8mm x 35 with washer and lock washer. Then install the stock bolt on the right side.

- Using the supplied templates and the appropriate size hole saw for upper front right mount (2”) and upper rear right mount (1 3/8”).   Full size templates are attached.

SUZUKI 650 BURGMAN MOUNTING INSTRUCTIONS

WIRING = BIKE SIDE:					WIRING = SIDECAR SIDE:

Black with white stripe = Ground 				Green = Right turn signal			   White with black stripe = Brake				Red = Brake

Green = Right turn signal					Black = Tail light

Gray = Tail light						White = Ground

- Access the stock wire harness where it passes near the right side seat hydraulic lift with supplied wire harness.

- Zip tie wire harness to frame and pass under upper rear mount.  The sidecar harness will pass through the same hole as the upper rear mount.

- Re-assemble body work in reverse order, starting with the right rear quarter panel.

- Brake line 30” with 90º banjo and quick disconnect for bike side of brake system.